THE LIST OF PURCHASING GOODS

Table 1.

| S/s | Name of the material | Unit of measurement | Quantity | Appropriate type, characteristics and and field of application | Standart |
| --- | --- | --- | --- | --- | --- |
| 1 | Side handle For transportation of 5 ton pipes (end catch) or equivalent | pieces | 4 | ZT-B-5.0 (bracket) Designation ZT-B-5.0 (bracket) Dimensions, mm A 35 C 180 H75 L 175 G30 Weight, kg 6.1 Designed to lift and carry pipes and similar loads in a horizontal position. | ГОСТ 25573-82 EN 13414 РД 10-33-93 |
| 2 | Earrings Lifting capacity 12 tons | pieces | 16 | Omega-shaped earrings (brackets) are a device designed for use with ropes and chains. Type 1 1/4 “ Lifting, tn 12 A mm 185 B mm 115 C mm 32 D mm 36 E mm 51 H mm 95 L mm 173 F mm 77 G mm 34 Weight, 1 unit / kg 4.9 | ГОСТ 25573-82 DIN EN 13889-2009 |
| 3 | Earrings Lifting capacity 25 tons | pieces | 8 | Type 1 3/4 “ Lifting, tn 25 A mm 265 B mm 161 C mm 46 D mm 50 E mm 69 H mm 143 L mm 225 F mm 102 G mm 46 Weight, 1 unit / kg 14.2 | ГОСТ 25573-82 DIN EN 13889-2009 Bow Shacle (омегообразный грузоподъёмный скоб ) |
| 4 | Earrings Lifting capacity 6.5 tons | pieces | 16 | Type 7.8 “; Type 7.8 “. Lifting, tn 6.5; Lifting, tn 6.5. A mm 132; A mm 132. B mm 81; B mm 81. C mm 23; C mm 23. D mm 26; D mm 26. E mm 35; E mm 35. H mm 68; H mm 68. L mm 117; L mm 117. F mm 52; F mm 52. G mm 23; G mm 23. Weight, 1 unit / kg 1.74; Weight, 1 unit / kg 1.74. | ГОСТ 25573-82 DIN EN 13889-2009 Bow Shacle (омегообразный грузоподъёмный скоб) |
| 5 | Hook (from wire material) STP 10.0 / 6000 or equivalent | pieces | 12 | . Carrying capacity -10 tn 2. Length - 6 m 3. polyester and polyamide -40 ° C to 100 ° C, 4. Polypropylene from -40 ° C to 80 ° C Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | РД 24-СЗК-01-01, EN1492-2:2000 Safety factor: 7: 1 |
| 6 | Hook (from wire material) STP 3.0 / 3000 or equivalent | pieces | 10 | . - 1. Cargo -3 tn - 2. Length-3 m - 3. polyester and polyamide - -40 ° C to 100 ° C, - 4. Polypropylene from -40 ° C to 80 ° C - Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with crane | РД 24-СЗК-01-01, EN1492-2:2000 Safety factor: 7: 1 |
| 7 | Hook (from wire material) STP 5.0 / 3000 or equivalent | pieces | 10 | - 1. Cargo -5 tn - 2. Length-3 m - 3. polyester and polyamide - -40 ° C to 100 ° C, - 4. Polypropylene from -40 ° C to 80 ° C - Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | РД 24-СЗК-01-01, EN1492-2:2000 Safety factor: 7: 1 |
| 8 | Hook (from wire material) STP 8.0 / 4000 or equivalent | pieces | 12 | - . Cargo -8 tn - 2. Length-4 m - 3. polyester and polyamide - -40 ° C to 100 ° C, - 4. Polypropylene from -40 ° C to 80 ° C - Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | РД 24-СЗК-01-01, EN1492-2:2000 Safety factor: 7: 1 |
| 9 | Hook (from wire material) STP- 8.0 / 6000 or equivalent | pieces | 12 | 1. Cargo -8 tn 2. Length-6 m 3. polyester and polyamide -40 ° C to 100 ° C, 4. Polypropylene from -40 ° C to 80 ° C Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | РД 24-СЗК-01-01, EN1492-2:2000 Safety factor: 7: 1 |
| 10 | Steel loop 1SK-10.0 / 1700 or equivalent | pieces | 10 | Brand of steel - StZsp Cargo -10 tn Length - 1.7 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 11 | Steel loop 1SK-10.0 / 6000 or equivalent | pieces | 18 | Brand of steel - StZsp Cargo -10 tn Length-6 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 12 | Steel loop 1SK-10.0 / 8000 or equivalent | pieces | 12 | Brand of steel - StZsp Cargo -10 tn Length-8 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 13 | Steel loop 1SK-12.5 / 6000 or equivalent | pieces | 12 | Brand of steel - StZsp Cargo -12.5 tn Length-6 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 14 | Steel loop 1SK-12.5 / 8000 or equivalent | pieces | 12 | Brand of steel - StZsp Cargo -12.5 tn Length-8 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 15 | Steel loop 1SK-5.0 / 1500 or equivalent | pieces | 12 | Brand of steel - StZsp Cargo -5 tn Length-1.5 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 16 | Steel loop 1SC- 5/2000 or equivalent | pieces | 12 | Brand of steel - StZsp Cargo -5 tn Length-2 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 17 | Steel loop 1SC- 5,0 / 6000 or equivalent | pieces | 12 | Brand of steel - StZsp Cargo -5 tn Length-6 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 18 | Steel loop 1SK-5.0 / 8000 or equivalent | pieces | 12 | rand of steel - StZsp Cargo -5 tn Length-8 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 19 | Steel loop 1SK-6.3 / 3000 or equivalent | pieces | 12 | Brand of steel - StZsp Cargo -6.3 tn Length-3 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 20 | Steel loop 1SK-6.3 / 8000 or equivalent | pieces | 12 | Brand of steel - StZsp Cargo -6.3 tn Length-8 m Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes. | ГОСТ 25573-82 Safety factor: 6: 1 |
| 21 | Hand ton 5 tons | pieces | 10 | Lifting capacity: 5 tons (chain length 10 meters) Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes, repair of engines and equipment. | ГОСТ 1106-74 ГОСТ 6899-75 ГОСТ 12.2.003-91 EN 13157:2004 Explosion-proof Ex d ll CT4 ГОСТ IEC 60079-10-1-2011 (ГОСТ Р МЭК 60079-10-1-2008) ATEX 114:2016 |
| 22 | 3 tons of hand hemp | pieces | 8 | ting capacity: 3 tons (chain length 8 meters) Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes, repair of engines and equipment. Standart - ГОСТ 1106-74 . | ГОСТ 1106-74 ГОСТ 6899-75 ГОСТ 12.2.003-91 EN 13157:2004 Explosion-proof Ex d ll CT4 ГОСТ IEC 60079-10-1-2011 (ГОСТ Р МЭК 60079-10-1-2008) ATEX 114:2016 |
| 23 | Traction mechanism Y3,2 or equivalent | pieces | 4 | Lifting capacity: 3.2 tons (chain length 8 meters) Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes, repair of engines and equipment. | ГОСТ 1106-74 ГОСТ 6899-75 ГОСТ 12.2.003-91 EN 13157:2004 Explosion-proof Ex d ll CT4 ГОСТ IEC 60079-10-1-2011 (ГОСТ Р МЭК 60079-10-1-2008) ATEX 114:2016 |
| 24 | hand hemp 1 TON | pieces | 4 | Lifting capacity: 1 ton (chain length 8 meters) Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes, repair of engines and equipment. | ГОСТ 1106-74 ГОСТ 6899-75 ГОСТ 12.2.003-91 EN 13157:2004 Explosion-proof Ex d ll CT4 ГОСТ IEC 60079-10-1-2011 (ГОСТ Р МЭК 60079-10-1-2008) ATEX 114:2016 |
| 25 | hand hemp 500kq | pieces | 4 | Lifting capacity: 0.5 tons (chain length 8 meters) Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes, repair of engines and equipment.. | ГОСТ 1106-74 ГОСТ 6899-75 ГОСТ 12.2.003-91 EN 13157:2004 Explosion-proof Ex d ll CT4 ГОСТ IEC 60079-10-1-2011 (ГОСТ Р МЭК 60079-10-1-2008) ATEX 114:2016 |
| 26 | 1.5 tons of hand hemp | pieces | 4 | Lifting capacity: 1.5 tons (chain length 8 meters) Area to be used: It is used in the "Ethylene Polyethylene" plant for lifting loads with cranes, repair of engines and equipment.. | ГОСТ 1106-74 ГОСТ 6899-75 ГОСТ 12.2.003-91 EN 13157:2004 Explosion-proof Ex d ll CT4 ГОСТ IEC 60079-10-1-2011 (ГОСТ Р МЭК 60079-10-1-2008) ATEX 114:2016 |
| 27 | Traction mechanism 3 tons manually operated | pieces | 6 | Traction mechanism Magnus-Profi WRP-3200 for use in warehouses, construction sites, as well as in cars. Designed to lift and pull loads with a maximum weight of 3200 kg. It is characterized by high working resource and resistance to increasing loads. Provides comfortable handling and a 20-meter sturdy cable. | ТР ТС 010/2011 ISO 9001:2015 ТУ 3173-002-39712863-2015 |
| 28 | action mechanism (tuapsinka) 5 tons | pieces | 6 | action mechanism Magnus-Profi WRP-3200 for use in warehouses, construction sites, as well as in cars. Designed to lift and pull loads with a maximum weight of 5400 kg. It is characterized by high working resource and resistance to increasing loads. Provides comfortable handling and a 20-meter sturdy cable. | ТР ТС 010/2011 ISO 9001:2015 ТУ 3173-002-39712863-2015 |
| 29 | Lifting mechanism (jack) 80 tons | pieces | 2 | Lifting capacity: 80 tons Area to be used: Used in the repair of railway tanks at the Ethylene Polyethylene Plant. ISO 9001: 2000 |  |